Weather Monitoring System

Using Arduino Uno

Project Documentation

DHT11 Sensor + 16x2 LCD with I2C Interface

# 1. Introduction

The Arduino-Based Weather Monitoring System is an embedded systems project that measures environmental temperature and humidity using a DHT11 sensor and displays the readings on a 16x2 LCD via I2C communication.

The system continuously reads sensor data every 3 seconds and updates the LCD display. A startup splash screen shows "Weather Monitor" for 2 seconds on power-on. If the sensor fails to respond, the display shows a "Sensor Error!" message instead of invalid data.

Powered by a 9V battery, the system is fully portable and suitable for indoor classroom or home use. This project introduces students to digital sensors, I2C communication, library usage, and real-time data display.

# 2. Objectives

- Measure real-time temperature using the DHT11 sensor via digital pin D2

- Measure real-time relative humidity using the same DHT11 sensor

- Display sensor readings on a 16x2 LCD using I2C communication at address 0x27

- Handle sensor read failures gracefully with an on-screen error message

- Show a startup splash screen on power-on for 2 seconds

- Refresh display data every 3 seconds using delay(3000)

- Understand library-based sensor and display interfacing in Arduino

# 3. Learning Outcomes

After completing this project, students will be able to:

- Understand the working principle of the DHT11 temperature and humidity sensor

- Use #include to import and use external Arduino libraries

- Initialize and communicate with an I2C LCD using LiquidCrystal_I2C

- Read float sensor values using dht.readTemperature() and dht.readHumidity()

- Use isnan() to detect and handle invalid sensor readings

- Control LCD cursor position using lcd.setCursor(col, row)

- Display special characters such as the degree symbol using lcd.write(byte(223))

- Build a complete, portable embedded system project

# 4. Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| DHT11 Temperature & Humidity Sensor | 1 |
| 16x2 LCD Display | 1 |
| I2C LCD Module (PCF8574) | 1 |
| Breadboard | 1 |
| Jumper Wires | Several |
| USB Cable | 1 |
| 9V Battery | 1 |
| Battery Connector | 1 |
| Enclosure Box (Optional) | 1 |

# 5. Estimated Cost

| Item | Approx. Cost (NPR) |
| --- | --- |
| Arduino Uno | 1,800 |
| DHT11 Sensor | 350 |
| LCD 16x2 | 450 |
| I2C Module | 200 |
| Breadboard | 250 |
| Jumper Wires | 200 |
| Battery & Clip | 300 |
| Enclosure | 600 |
| Miscellaneous | 500 |

Estimated Total Cost: NPR 5,000 – 6,500

# 6. Working Principle

The DHT11 sensor measures surrounding temperature and humidity using a capacitive humidity element and a thermistor. It communicates via a single-wire digital protocol on pin D2.

The Arduino reads the sensor using the DHT library functions dht.readTemperature() and dht.readHumidity(), which return float values. The isnan() function checks whether the returned values are valid numbers before displaying them.

The 16x2 LCD communicates with the Arduino via I2C using only two wires (SDA on A4, SCL on A5). The I2C address is set to 0x27, though some modules use 0x3F.

Display Format (from sketch)

| LCD Row | Content | Example |
| --- | --- | --- |
| Row 0 (top) | Temp: <value> degree C | Temp:28.4°C |
| Row 1 (bottom) | Hum : <value>% | Hum :61.0% |

Temperature is printed with 1 decimal place using lcd.print(temperature, 1). The degree symbol is produced by lcd.write(byte(223)), which sends the LCD's custom character code for the degrees character.

# 7. Block Diagram

+--------------------+

|    DHT11 Sensor    |

|  (Digital Pin D2) |

+---------+----------+

|

Single-wire digital

|

v

+---------------+

|  Arduino Uno  |

+-------+-------+

|

I2C (SDA=A4, SCL=A5)

|

v

+--------------------+

|  16x2 LCD Display  |

|   (addr: 0x27)     |

+--------------------+

Powered by 9V Battery

# 8. Circuit Connections

DHT11 Sensor

| DHT11 Pin | Arduino Pin | Variable / Define |
| --- | --- | --- |
| VCC | 5V | — |
| GND | GND | — |
| DATA | Digital Pin 2 | DHTPIN (defined as 2) |

LCD I2C Module

| LCD Pin | Arduino Pin | Notes |
| --- | --- | --- |
| VCC | 5V | Power supply |
| GND | GND | Common ground |
| SDA | A4 | I2C data line |
| SCL | A5 | I2C clock line |

# 9. Software Requirements

| Library | Purpose | Install Via |
| --- | --- | --- |
| Wire.h | Built-in I2C communication | Built into Arduino IDE |
| LiquidCrystal_I2C | Controls the 16x2 LCD over I2C | Library Manager → search LiquidCrystal I2C |
| DHT.h | Reads DHT11/DHT22 sensor data | Library Manager → search DHT sensor library (Adafruit) |
| Adafruit Unified Sensor | Dependency for DHT library | Library Manager → search Adafruit Unified Sensor |

Installing Libraries

- Open Arduino IDE

- Go to Sketch → Include Library → Manage Libraries

- Search for and install: DHT sensor library by Adafruit

- Search for and install: Adafruit Unified Sensor

- Search for and install: LiquidCrystal I2C by Frank de Brabander

# 10. Source Code

The complete Arduino sketch is shown below. It uses three external libraries: Wire (built-in), LiquidCrystal_I2C, and DHT.

Library Includes & Object Declarations

Three libraries are included. The LCD object is created with I2C address 0x27 for a 16x2 display. The DHT object is bound to pin D2 as a DHT11 type:

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <DHT.h>

// ----------------------

// Pin Definitions

// ----------------------

#define DHTPIN 2

#define DHTTYPE DHT11

// ----------------------

// LCD Address

// Usually 0x27 or 0x3F

// ----------------------

LiquidCrystal_I2C lcd(0x27, 16, 2);

DHT dht(DHTPIN, DHTTYPE);

setup() — Initialization

Initializes the LCD (lcd.init(), lcd.backlight()), starts the DHT sensor (dht.begin()), shows a 2-second splash screen, then hands off to loop():

void setup()

{

lcd.init();

lcd.backlight();

dht.begin();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(" Weather");

lcd.setCursor(0, 1);

lcd.print(" Monitor");

delay(2000);

}

loop() — Sensor Read & Display

Reads humidity and temperature as floats. Validates with isnan(). On success, clears the LCD and prints formatted data with the degree symbol (byte 223). Refreshes every 3 seconds:

void loop()

{

float humidity    = dht.readHumidity();

float temperature = dht.readTemperature();

// Check if reading failed

if (isnan(humidity) || isnan(temperature))

{

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Sensor Error!");

delay(2000);

return;

}

lcd.clear();

// First Line

lcd.setCursor(0, 0);

lcd.print("Temp:");

lcd.print(temperature, 1);

lcd.write(byte(223)); // Degree symbol

lcd.print("C");

// Second Line

lcd.setCursor(0, 1);

lcd.print("Hum :");

lcd.print(humidity, 1);

lcd.print("%");

delay(3000);

}

Full Sketch

Complete sketch for copy/paste or direct upload to Arduino IDE:

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <DHT.h>

// ----------------------

// Pin Definitions

// ----------------------

#define DHTPIN 2

#define DHTTYPE DHT11

// ----------------------

// LCD Address

// Usually 0x27 or 0x3F

// ----------------------

LiquidCrystal_I2C lcd(0x27, 16, 2);

DHT dht(DHTPIN, DHTTYPE);

void setup()

{

lcd.init();

lcd.backlight();

dht.begin();

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(" Weather");

lcd.setCursor(0, 1);

lcd.print(" Monitor");

delay(2000);

}

void loop()

{

float humidity    = dht.readHumidity();

float temperature = dht.readTemperature();

// Check if reading failed

if (isnan(humidity) || isnan(temperature))

{

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Sensor Error!");

delay(2000);

return;

}

lcd.clear();

// First Line

lcd.setCursor(0, 0);

lcd.print("Temp:");

lcd.print(temperature, 1);

lcd.write(byte(223)); // Degree symbol

lcd.print("C");

// Second Line

lcd.setCursor(0, 1);

lcd.print("Hum :");

lcd.print(humidity, 1);

lcd.print("%");

delay(3000);

}

# 11. Code Walkthrough

| Line / Block | Purpose |
| --- | --- |
| #define DHTPIN 2 | Assigns DHT11 data pin to D2 |
| #define DHTTYPE DHT11 | Specifies sensor model for the DHT library |
| LiquidCrystal_I2C lcd(0x27, 16, 2) | Creates LCD object at I2C address 0x27, 16 columns, 2 rows |
| DHT dht(DHTPIN, DHTTYPE) | Creates DHT sensor object bound to pin D2 |
| lcd.init() | Initializes the I2C LCD hardware |
| lcd.backlight() | Turns on the LCD backlight |
| dht.begin() | Initializes the DHT11 sensor |
| lcd.clear() | Clears all characters from the LCD screen |
| lcd.setCursor(col, row) | Positions the cursor — column 0-15, row 0-1 |
| dht.readHumidity() | Returns relative humidity as a float (0.0–100.0) |
| dht.readTemperature() | Returns temperature in Celsius as a float |
| isnan(value) | Returns true if value is not a valid number (failed read) |
| lcd.print(temperature, 1) | Prints float with 1 decimal place to LCD |
| lcd.write(byte(223)) | Writes degree symbol using LCD character code 223 |
| delay(3000) | Waits 3 seconds before next sensor read cycle |

# 12. Algorithm

| Step | Action |
| --- | --- |
| 1 | Power on — Arduino initializes |
| 2 | setup(): lcd.init(), lcd.backlight(), dht.begin() |
| 3 | setup(): show splash screen "Weather / Monitor" for 2 seconds |
| 4 | loop(): call dht.readHumidity() and dht.readTemperature() |
| 5 | Check: isnan(humidity) OR isnan(temperature)? |
| 6a | YES (error): lcd.clear(), print "Sensor Error!", delay 2s, return |
| 6b | NO (valid): lcd.clear(), print Temp on row 0, print Hum on row 1 |
| 7 | delay(3000) — wait 3 seconds |
| 8 | Return to Step 4 — repeat indefinitely |

# 13. Expected Output

Startup Splash Screen (2 seconds)

| LCD Row | Displayed Text |
| --- | --- |
| Row 0 | Weather |
| Row 1 | Monitor |

Normal Operation

| LCD Row | Displayed Text | Example |
| --- | --- | --- |
| Row 0 | Temp:<value>°C | Temp:28.4°C |
| Row 1 | Hum :<value>% | Hum :61.0% |

Sensor Error State

| LCD Row | Displayed Text |
| --- | --- |
| Row 0 | Sensor Error! |
| Row 1 | (blank) |

Readings update every 3 seconds as controlled by delay(3000) in the loop. On sensor failure, the error message shows for 2 seconds before retrying.

# 14. Finding Your LCD I2C Address

The sketch uses I2C address 0x27 by default. Some LCD I2C modules ship with address 0x3F. If the LCD shows nothing, run an I2C scanner sketch to find the correct address, then update this line:

LiquidCrystal_I2C lcd(0x27, 16, 2);  // Change 0x27 to your address

I2C Scanner Sketch

#include <Wire.h>

void setup() {

Wire.begin();

Serial.begin(9600);

for (byte addr = 1; addr < 127; addr++) {

Wire.beginTransmission(addr);

if (Wire.endTransmission() == 0) {

Serial.print("Found: 0x");

Serial.println(addr, HEX);

}

}

}

void loop() {}

Upload this sketch, open Serial Monitor at 9600 baud, and note the address printed. Use that value in the main sketch.

# 15. Features

- Real-time temperature and humidity monitoring via DHT11 every 3 seconds

- I2C LCD interface — only 2 wires (SDA, SCL) needed for display

- Startup splash screen on power-on

- Graceful sensor error handling with on-screen message

- Temperature displayed with 1 decimal place and degree symbol

- Humidity displayed with 1 decimal place and % unit

- Portable — powered by 9V battery

- Beginner-friendly code with clear comments

# 16. Applications

- School and college science exhibitions

- Classroom weather station demonstrations

- Home indoor climate monitoring

- Laboratory environment monitoring

- Agricultural awareness and education

- STEM education and embedded systems training

# 17. Advantages

- Low cost (NPR 5,000 – 6,500)

- Easy to assemble — minimal components

- Portable — 9V battery powered

- I2C reduces wiring to just 2 data lines for the LCD

- Built-in error detection prevents display of invalid readings

- Accurate enough for educational use

- Real-time display with automatic 3-second refresh

# 18. Limitations

- DHT11 accuracy: ±2°C temperature, ±5% humidity — limited for precision use

- Indoor use recommended — DHT11 not suitable for outdoor or harsh environments

- No data logging — readings are not stored between power cycles

- No internet connectivity or remote monitoring

- No historical weather records or trend display

- Serial Monitor output not included in the sketch — add Serial.println() for debugging

# 19. Future Improvements

- Replace DHT11 with DHT22 for higher accuracy (±0.5°C, ±2% humidity)

- Add Wi-Fi using ESP8266 or upgrade to ESP32 for wireless monitoring

- Upload readings to cloud platforms such as ThingSpeak or Blynk

- Add SD card module for data logging and historical records

- Display weather trend graphs on an OLED or TFT screen

- Add RTC (DS3231) to timestamp each reading

- Include BMP280 pressure sensor for full weather station capability

- Add Serial.println() output for PC-side logging and debugging

- Power via solar panel and rechargeable LiPo for outdoor deployment

# 20. Safety Precautions

- Verify all wiring before powering the circuit

- Avoid short circuits on the breadboard

- Keep the DHT11 sensor dry — do not expose to condensation or rain

- Do not exceed Arduino's 5V/3.3V input voltage limits

- Disconnect power before modifying or changing any connections

- Confirm I2C address (0x27 or 0x3F) before upload to avoid blank LCD

# 21. Conclusion

The Arduino-Based Weather Monitoring System demonstrates how a digital sensor and I2C display can be combined into a complete, portable embedded system. The sketch uses three key libraries — DHT, LiquidCrystal_I2C, and Wire — covering important concepts including library-based sensor communication, I2C protocol, float data handling, error detection with isnan(), and custom LCD character output.

The built-in sensor error handling makes the system robust, while the splash screen and formatted output give it a polished feel. With straightforward upgrades such as an ESP32 or DHT22, this project can evolve into a full-featured wireless weather station.

Arduino Weather Monitoring System — Project Documentation v1.0